# PRE-PURCHASE INSPECTION CHECKLIST

1986 Jeep Comanche 4x4 | 2.5L AMC 4-Cyl | 5-Speed Manual

WINTER DRIVING SAFETY FOCUS | Mountain Road Daily Driver | Mission-Critical Farm Access

| VIN: ________________ | Mileage: ____________ | Date: ______________ | Mechanic: ___________ |
| --- | --- | --- | --- |

| 🚨 CRITICAL SAFETY: BRAKING SYSTEM (Mountain Road Priority) |  |  |  |
| --- | --- | --- | --- |
| ✓ | Inspection Item | Result | Notes / Findings |
| ☐ | Master cylinder fluid level and condition | P / F / NA | Check for dark/contaminated fluid |
| ☐ | Brake pedal feel - firm, no spongy feel | P / F / NA | Pump test: should not sink |
| ☐ | Front brake pad thickness (min 4mm) | P / F / NA | Measure both sides |
| ☐ | Rear brake shoe thickness (min 2mm) | P / F / NA | Drum brakes on rear |
| ☐ | Brake rotors - no deep grooves/warping | P / F / NA | Check for heat damage |
| ☐ | Brake drums - no scoring/cracks | P / F / NA | Inspect inside surface |
| ☐ | All brake lines - no rust/leaks/bulging | P / F / NA | Follow lines to each wheel |
| ☐ | Brake hoses - no cracking/swelling | P / F / NA | Flex test for deterioration |
| ☐ | Wheel cylinders - no leakage | P / F / NA | Check behind drums |
| ☐ | Parking/emergency brake operation | P / F / NA | Hold vehicle on incline |
| ☐ | ABS (if equipped) - warning light test | P / F / NA | May not have ABS |
| 🚨 CRITICAL SAFETY: STEERING SYSTEM |  |  |  |
| ✓ | Inspection Item | Result | Notes / Findings |
| ☐ | Power steering fluid level | P / F / NA | Check with engine running |
| ☐ | Power steering pump - no whine/leaks | P / F / NA | Listen during turns |
| ☐ | Steering box - no play/leaks | P / F / NA | Check mounting bolts |
| ☐ | Tie rod ends - no play/torn boots | P / F / NA | Shake wheels side-to-side |
| ☐ | Drag link and center link condition | P / F / NA | Inspect for wear |
| ☐ | Ball joints - upper and lower | P / F / NA | Pry test with bar |
| ☐ | Steering wheel play (max 2 inches) | P / F / NA | Engine running test |
| ☐ | Steering column - no looseness | P / F / NA | Check mounting and U-joints |
| 🔧 4WD SYSTEM (Essential for Snow/Ice) |  |  |  |
| ✓ | Inspection Item | Result | Notes / Findings |
| ☐ | Transfer case fluid level and condition | P / F / NA | NP231 or NP207 - check tag |
| ☐ | 4WD engagement - 4H shifts smoothly | P / F / NA | Test at low speed |
| ☐ | 4WD engagement - 4L shifts smoothly | P / F / NA | Test stopped or crawling |
| ☐ | Front axle engagement (vacuum or manual) | P / F / NA | Verify hubs lock |
| ☐ | Front/rear differential fluid level | P / F / NA | Check both axles |
| ☐ | CV/U-joints - no play or clicking | P / F / NA | Front axle shafts |
| ☐ | Transfer case linkage adjustment | P / F / NA | Ensure full engagement |
| ☐ | No grinding in any 4WD mode | P / F / NA | Listen during operation |
| ⚙️ ENGINE & DRIVETRAIN (2.5L AMC I4) |  |  |  |
| ✓ | Inspection Item | Result | Notes / Findings |
| ☐ | Cold start behavior | P / F / NA | Already noted: starts well |
| ☐ | Oil level and condition | P / F / NA | Note: leak at oil pressure sender |
| ☐ | Oil pressure at idle (min 13 psi) | P / F / NA | Use gauge if possible |
| ☐ | Coolant level and condition | P / F / NA | 50/50 mix for winter |
| ☐ | Coolant hoses - no cracks/swelling | P / F / NA | Upper, lower, heater hoses |
| ☐ | Thermostat operation (195°F) | P / F / NA | Verify it opens |
| ☐ | Water pump - no leaks/bearing play | P / F / NA | Check weep hole |
| ☐ | Radiator - no leaks, cap holds pressure | P / F / NA | Pressure test recommended |
| ☐ | Serpentine/V-belt condition | P / F / NA | No cracks, proper tension |
| ☐ | Battery - load test and terminals | P / F / NA | Critical for cold starts |
| ☐ | Alternator output (13.5-14.5V) | P / F / NA | Test with multimeter |
| ☐ | Starter operation | P / F / NA | Already noted: starts first crank |
| ☐ | Exhaust manifold - no cracks/leaks | P / F / NA | Note: exhaust smell in cab |
| ☐ | Exhaust system - no leaks/rust-through | P / F / NA | Inspect entire system |
| ⚙️ TRANSMISSION & CLUTCH (AX5 5-Speed) |  |  |  |
| ✓ | Inspection Item | Result | Notes / Findings |
| ☐ | Clutch pedal free play (1-1.5 inches) | P / F / NA | Adjust if needed |
| ☐ | Clutch engagement - smooth, no slipping | P / F / NA | Hill test recommended |
| ☐ | All forward gears shift smoothly | P / F / NA | Already noted: good |
| ☐ | Reverse engagement | P / F / NA | Already noted: slightly sticky |
| ☐ | Transmission fluid level (ATF or GL-4) | P / F / NA | Check with AX5 spec |
| ☐ | No grinding or popping out of gear | P / F / NA | Test under load |
| ☐ | Clutch hydraulics - no leaks | P / F / NA | Master and slave cylinder |
| 🛞 SUSPENSION & TIRES |  |  |  |
| ✓ | Inspection Item | Result | Notes / Findings |
| ☐ | Shock absorbers - bounce test | P / F / NA | Should settle in 1-2 bounces |
| ☐ | Leaf springs - no cracks/broken leaves | P / F / NA | Rear suspension |
| ☐ | Control arm bushings | P / F / NA | Check for deterioration |
| ☐ | Sway bar links and bushings | P / F / NA | Front and rear if equipped |
| ☐ | Wheel bearings - no play/noise | P / F / NA | Rotate and check |
| ☐ | Tire condition - Falken Wild Peak AT | P / F / NA | Already noted: NEW |
| ☐ | Tire pressure and tread depth | P / F / NA | Set for cold weather |
| ☐ | Spare tire and jack present | P / F / NA | Verify condition |
| 💡 ELECTRICAL & SAFETY SYSTEMS |  |  |  |
| ✓ | Inspection Item | Result | Notes / Findings |
| ☐ | Headlights - high and low beam | P / F / NA | Critical for winter darkness |
| ☐ | Tail lights and brake lights | P / F / NA | All bulbs working |
| ☐ | Turn signals front and rear | P / F / NA | Including hazards |
| ☐ | Windshield wipers - operation/blades | P / F / NA | Replace blades for winter |
| ☐ | Windshield washer fluid and pump | P / F / NA | Use winter-rated fluid |
| ☐ | Heater/defroster operation | P / F / NA | Must defrost windshield |
| ☐ | Blower motor all speeds | P / F / NA | Heat distribution |
| ☐ | Horn operation | P / F / NA | Safety requirement |
| ☐ | Fuel gauge accuracy | P / F / NA | Test with known fill |
| ☐ | Ignition switch - engine shuts off with key | P / F / NA | Known issue: won't shut off |
| 🚗 CAB SAFETY & SEALING |  |  |  |
| ✓ | Inspection Item | Result | Notes / Findings |
| ☐ | Floor pan integrity - no rust-through | P / F / NA | Check under mats |
| ☐ | Exhaust intrusion source | P / F / NA | Already noted: exhaust smell |
| ☐ | Firewall seals and grommets | P / F / NA | Prevent fumes entering |
| ☐ | Door seals - keep out water/cold | P / F / NA | Check all seals |
| ☐ | Window operation | P / F / NA | Must seal properly |
| ☐ | Seat belts - function and condition | P / F / NA | All occupant positions |
| ☐ | Hood latch/release operation | P / F / NA | Known issue: sticky/difficult |

| CRITICAL ISSUES FOUND: 1. ________________________________ 2. ________________________________ 3. ________________________________ 4. ________________________________ | ESTIMATED REPAIR COSTS: Immediate (before driving): $________ Within 30 days: $________ Spring/Summer: $________ TOTAL: $________ |
| --- | --- |

OVERALL ASSESSMENT:  ☐ SAFE FOR IMMEDIATE WINTER USE    ☐ NEEDS REPAIRS BEFORE USE    ☐ NOT RECOMMENDED

Mechanic Signature: ________________________    Date: ____________    Shop: ________________________