# REPAIR COST BREAKDOWN & BUDGET

1986 Jeep Comanche 4x4 | 2.5L AMC 4-Cyl | 5-Speed Manual

Prioritized for Immediate Winter Driving | Mountain Road Farm Access

KNOWN ISSUES FROM SELLER (Per Buyer Description): • Rough idle on cold start (runs well once warm) • Exhaust smell in cab (potential exhaust leak/manifold issue) • Engine does not shut off when key turned to OFF • Reverse gear slightly sticky • Oil leak at oil pressure sending unit • Hood latch/release sticky - doesn't open easily • New valve cover gasket installed ¦ New Falken Wild Peak AT tires installed

| 🚨 PRIORITY 1: MUST COMPLETE BEFORE DAY ONE (Safety Critical) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Repair Item | Parts | Parts $ | Labor $ | Total |
| Exhaust leak repair CRITICAL: CO poisoning risk | Exhaust manifold gasket, donut gasket, clamps | $85 | $150 | $235 |
| Exhaust manifold inspection Inspect for cracks - add $180 if replacement needed | May need manifold if cracked | $0 | $50 | $50 |
| Oil pressure sending unit Known leak - easy fix | OEM sender unit | $25 | $30 | $55 |
| Ignition switch / shut-off repair Engine won't shut off with key | Ignition switch or idle stop solenoid | $50 | $75 | $125 |
| Brake fluid flush If fluid is dark/old | DOT 3/4 brake fluid | $15 | $60 | $75 |
| Brake inspection/adjustment Estimate - may need parts | Pads/shoes if <4mm/<2mm | $120 | $80 | $200 |
| Battery load test/replace Critical for cold starts | Group 34 battery | $150 | $0 | $150 |
| Coolant flush & fill Ensure freeze protection | 2 gal 50/50 antifreeze | $30 | $45 | $75 |
| Windshield wiper blades Essential for snow | 18" + 18" winter blades | $40 | $0 | $40 |
| Windshield washer fluid | Winter-rated -20°F fluid | $10 | $0 | $10 |
| All lights inspection/bulbs Ensure all safety lights work | Assorted bulbs | $25 | $30 | $55 |
| 4WD system verification Must engage properly for snow | Transfer case fluid top-off | $20 | $60 | $80 |
| PRIORITY 1 SUBTOTAL: |  |  |  | $1095-1275 |

| ⚠️ PRIORITY 2: COMPLETE WITHIN 2 WEEKS (Reliability) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Repair Item | Parts | Parts $ | Labor $ | Total |
| Hood latch/release repair Known issue: sticky/difficult | Lube, cable adjust, or replace | $25 | $25 | $50 |
| Transmission fluid change AX5 spec - helps sticky reverse | 2.5 qt GL-4 75W-90 or ATF | $35 | $45 | $80 |
| Transfer case fluid change | 2 qt ATF (NP231) | $20 | $45 | $65 |
| Front/rear differential fluid | 4 qt 80W-90 gear oil | $40 | $60 | $100 |
| Serpentine/accessory belt Preventive replacement | Gates or Continental belt | $35 | $30 | $65 |
| Heater hoses inspection Critical for winter heat | Replace if cracked | $40 | $60 | $100 |
| Thermostat replacement Ensure proper warm-up | 195°F thermostat + gasket | $20 | $60 | $80 |
| Fuel filter replacement Preventive maintenance | In-line fuel filter | $15 | $30 | $45 |
| Air filter replacement | Panel air filter | $20 | $0 | $20 |
| Spark plugs If unknown service history | 4x Champion RC12YC | $15 | $40 | $55 |
| PCV valve Often overlooked | PCV valve | $8 | $15 | $23 |
| PRIORITY 2 SUBTOTAL: |  |  |  | $683 |

| 📋 PRIORITY 3: COMPLETE WITHIN 30 DAYS (Durability) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Repair Item | Parts | Parts $ | Labor $ | Total |
| Steering system inspection May need front end work | Tie rod ends if worn | $80 | $120 | $200 |
| Ball joint inspection If play detected | Upper/lower if worn | $120 | $200 | $320 |
| U-joints inspection Replace if any play | Front/rear driveshaft | $60 | $80 | $140 |
| Shock absorbers If bounce test fails | 4x Monroe or KYB shocks | $200 | $160 | $360 |
| Wheel bearing repack If noise or play | Grease, seals | $30 | $120 | $150 |
| Clutch adjustment/inspection Ensure proper engagement | Clutch hydraulic fluid | $10 | $45 | $55 |
| PRIORITY 3 SUBTOTAL: |  |  |  | $1225 |

| 🌡️ PRIORITY 4: SPRING/SUMMER PREP (Heat Management - Defer) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Repair Item | Parts | Parts $ | Labor $ | Total |
| Radiator flush/inspection Only if issues found | Radiator if leaking | $180 | $150 | $330 |
| Water pump replacement Preventive if high mileage | Water pump + gasket | $85 | $120 | $205 |
| Cooling fan clutch If overheating occurs | Thermal fan clutch | $75 | $60 | $135 |
| A/C system inspection Not critical for farm use | Refrigerant, compressor eval | $50 | $100 | $150 |
| Radiator hoses Replace if swollen/cracked | Upper + lower hoses | $50 | $45 | $95 |
| PRIORITY 4 SUBTOTAL: |  |  |  | $915 |

| BUDGET SUMMARY |  |  |
| --- | --- | --- |
| Priority 1 - Before Day One (MANDATORY) | $1,095 - $1,275 | DO FIRST |
| Priority 2 - Within 2 Weeks | $683 |  |
| Priority 3 - Within 30 Days | $1,225 |  |
| Priority 4 - Spring/Summer (Deferred) | $915 | CAN WAIT |
| TOTAL POTENTIAL INVESTMENT: | $3,918 - $4,098 |  |

NEGOTIATION NOTES: • Known exhaust leak (cab smell) + oil leak = documented safety/reliability issues • Minimum $1,095-1,275 required immediately for safe winter operation • Credit for new tires (~$800 value) already applied by seller • Request inspection findings be deducted from asking price • 1986 Comanche with 2.5L/5-speed is durable but 38+ years old - expect repairs Suggested negotiation: Asking price minus Priority 1 costs = Fair purchase price

Note: All prices are estimates based on typical repair costs. Actual costs may vary by region and shop rates. Parts prices assume quality aftermarket; OEM/Mopar parts may cost more.